| Integrated Granulation Line | URS No.: URSPD023 | Supersedes: | N/A |
| --- | --- | --- | --- |
|  | Rev: 00 | Effective Date: |  |
| USER REQUIREMENT SPECIFICATION OF INTEGRATED GRANULATION LINE & IBC BIN |  |  |  |

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
| Sr. No. | ### Topics | Page No. |
| 1.0 | APPROVAL | 3 |
| 2.0 | INTRODUCTION | 4 |
| 3.0 | OVERVIEW | 4-5 |
| 4.0 | STATUTORY REGULATIONS | 5-6 |
| 5.0 | PROCESS | 6-12 |
| 6.0 | CLEANING REQUIREMENTS | 13-14 |
| 7.0 | GENERAL REQUIREMENT FUNCTIONS | 14-28 |
| 8.0 | INSTALLATION | 28-29 |
| 9.0 | HEALTH, SAFETY AND ENVIRONMENT | 30 |
| 10.0 | DOCUMENTATION / TRAINING | 31-32 |
| 11.0 | REFERENCES AND DEFINITIONS | 33 |
| 12.0 | ABBREVIATIONS | 33-34 |
| 13.0 | REVISION HISTORY | 34 |
| 14.0 | VENDOR ACCEPTANCE | 35 |

| 1.0 | #### APPROVAL PAGE The User requirement specifications for Integrated Granulation Line have been prepared, reviewed, and approved by concerns from the following area |  |  |  |
| --- | --- | --- | --- | --- |
| #### Prepared By |  |  |  |  |
| Department |  | Name & Designation | Signature | Date |
| Reviewed By |  |  |  |  |
| Department |  | Name & Designation | Signature | Date |
| Approved By |  |  |  |  |
| Department |  | Name & Designation | Signature | Date |

| ## 2.0 | ## INTRODUCTION |
| --- | --- |
|  | The URS has been initiated by Novugen Oncology Sdn Bhd and pertains to procurement of an Integrated Granulation Line. Being end user of proposed equipment, the user department has initiated the document, which has been reviewed by Engineering / Project, Quality Assurance and approved by Quality Assurance. This document is prepared to help suppliers to understand the user requirement for developing the functional specification and design specification. Each of approval by Novugen Oncology Sdn Bhd. will become a contractual agreement between suppliers and Novugen Oncology Sdn Bhd. The project is to be completed in two phases. In the first phase, a complete of the integrated granulation is to be designed and supplied with a provision of an additional integrated second Wet Mill and Fluidize Bed Processor. An additional Dry Mill may be considered only if it cannot support second system. |

| 3.0 | OVERVIEW |  |  |
| --- | --- | --- | --- |
| 3.1 | Name of System/Equipment/ Instrument/ Software | : | Integrated Granulation Line |
| 3.2 | System/Equipment/ Instrument/ Software | : | The process covered by this URS is for Integrated Granulation Line. Raw materials are dispensed and stored in an Intermediate Bulk Container (IBC) Bin or a plybottle (≤ 500L), which is automatically docked onto an auto discharge station mounted on the bowl cover of the high shear mixer. The powder then flows by gravity into the mixer bowl. Inside the high shear mixer, the materials are blended and granulated with the addition of a binder solution. Upon completion of granulation, the wet granules are discharged through a pneumatically operated side valve and transferred—via vacuum—through a wet mill into the Fluid Bed Processor (FBP) for drying. Once drying is complete, the dried product is discharged from the FBP through a hose into a hopper station. From there, the granules pass through a variable-speed dry mill and are collected in an Intermediate Bulk Container (IBC) for further processing. Alternatively, for products made using top-spray granulation, the raw materials are transferred straight into the Fluid Bed Processor (FBP) using a Polybottle/IBC Bin. The Polybottle/IBC Bin is connected to an auto discharge station, which helps move the powder into the FBP. Since a high shear mixer is not used in this process, a vacuum system is needed to pull the powder from the discharge hose into the FBP. The FBP carries out both granulation and drying in one step. After the process is complete, the dried granules are released through a discharge valve, passed through the lean phase hopper and dry mill, before collecting in an Intermediate Bulk Container (IBC). Milled granules in the IBC are to be taken for blending at the Bin Blender. If required, the granules shall be transferred to larger sized IBCs using the bin-to-bin transfer system. |
| 3.3 | Desired/ Suggested Capacity | : | Intermediate Bulk Container (IBC) Bin + frame with Buck Valve: 500L (maximum system weight should be < 130kg) – 5 units Bin Lifter and Positioning Device: 400kg Auto Discharge Station: 150mm valve size Granulation bowl: 250L – 300L Wet Milling Screen: 10mm Fluidized bed processor bowl: 250L – 300L Top Spray Mount Hopper Station: should accommodate entire dried granules from FBP (~250L – 300L) Dry Milling Screen: multiple sizes |
| 3.4 | Number of System/Equipment/ Instrument/ Software | : | 8 units of equipment together with an integrated system and software. - IBC Bins (500L) – 5 units - Rapid Mixer Granulator with detachable platform ladder - Solution preparation tank - Wet Mill (with provision of installing another additional unit) - Fluid Bed Processor (with provision of installing another additional unit) - Hopper station (wall mounted and height-adjustable) - Dry Mill (wall mounted and height-adjustable) - Bin Lifter and Positioning Device for IGL room |

| 4.0 | STATUTORY REGULATIONS |  |  |
| --- | --- | --- | --- |
| 4.1 | General Regulations |  |  |
| 4.2 | GMP standards | : | The equipment should be designed and manufactured in accordance with the latest editions and subsequent amendments of all relevant USFDA, EU and WHO. - EU: The Rules Governing Medicinal Products in European Union – Eudralex, volume IV ‘Good Manufacturing Practice’ Part 1 - EU GMP Guide, Annex 15 (Qualification and Validation) - US FDA: 21 CFR Parts 210 and 211 – cGMP in Manufacturing, Processing, Packing or Holding of Drugs and Finished Pharmaceuticals. - US FDA: 21 CFR Part 11 – Electronic Records; Electronic Signatures. |
| 4.3 | Local Regulations | : | Environmental Quality Act 1974 (Act 127) and all related regulations Occupational Safety and Health Act 1994 (Act 514) and all related regulations Factories and Machineries Act 1976 (Act 139) and related regulations Electricity Supply Act 1990 (Act 447) and all related regulations Software shall comply with GxP and 21 CFR Part 11 regulations. |
| 4.4 | EC directives and declarations | : | EC directives and declarations to be followed / issued in general. The supplier must ensure that all relevant directives are adhered to / for a given delivery. ATEX certified flame proof declaration should be provided where flame proof design is considered in manufacturing equipment. |
| 4.5 | Guidelines | : | The following guidelines for design should be consulted to help satisfy the cGMP’s. It should also comply with regulations of USFDA, EU standards, ISPE BASELINE GUIDE, VOLUME 2, PIC/S Guide to Good Manufacturing Practice for Medicinal Products and ISPE Good Practice Guide, Assessing the Particulate Containment Performance of Pharmaceutical Equipment. |
| 4.6 | 21 CFR Part 11 Compliance | : | The system should meet the requirement of 21 CFR part 11 compliance. |
| 4.7 | GAMP requirements | : | The system should meet the standard requirement GAMP 5, Good Automated Manufacturing Practice - Guide for Validation of Automated Systems in Pharmaceutical Manufacture. |

| 5.0 | PROCESS |
| --- | --- |
| 5.1 | Process Description |
|  | Binder is prepared from dispensed materials inside a solution preparation tank that is capable of handling both solvent and aqueous based binders. The binder is then sprayed onto the materials in the high shear mixer or the fluid bed processor (FBP), depending on the selected processes. Raw materials are dispensed and stored in a Polybottle/IBC Bin with a buck valve, that is then docked onto an auto discharge station mounted on the bowl cover of the high shear mixer. The powder then flows by gravity into the mixer bowl. Inside the high shear mixer, the materials are blended and granulated with the addition of a binder solution. Upon completion of granulation, the wet granules are discharged through a pneumatically operated side valve and transferred—via gravity or vacuum—through a wet mill into the FBP for drying. Once drying is complete, the dried product is discharged from the FBP through a discharge hose into a lean-phase hopper (LPH). The LPH will collect the granules from the FBP for further process. Once all the granules are collected in the hopper, the granules will pass through a variable-speed dry mill and are collected in an IBC for further processing. Alternatively, for products made using top-spray granulation, the raw materials are transferred straight into the Fluid Bed Processor (FBP) using a Polybottle/IBC Bin. The Polybottle/IBC Bin is connected to an auto discharge station, which helps move the powder into the FBP. Since a high shear mixer is not used in this process, a vacuum system is needed to pull the powder from the discharge hose into the FBP. The FBP carries out both granulation and drying in one step. After the process is completed, the dried granules are released through a discharge valve, into the LPH, then passed through the dry mill, and collected in an IBC for the next stage. The steps above are repeated for batches of multiple lots. All the systems should have a built in clean-in-place (CIP) system that can run concomitantly and independently of each other with a total washing time (four cycles of wash) of not exceeding 3 hours. All equipment shall be equipped with a rotating actuator spray ball for the cleaning process. All equipment shall incorporate ATEX-proof design for zone 20. On the HMI a recipe for the product is selected with validated parameters. For example, pump speed, impeller speed, inlet air flow, and milling speed. The operation, recipe and process control screen and controls shall be able to be configured as per client’s request. |
|  | Process Module; Description; Remarks. Raw Material Charging; Raw materials are transferred into the high shear mixer granulator using a polybottle/IBC Bin equipped with a buck valve. The polybottle/IBC Bin is connected to an automatic discharge station, allowing the powder to flow by gravity into the mixer for further processing.; Lifter shall be used for charging the polybottle/IBC Bin on the granulator.. Solution Preparation Tank with Transfer Pump; To prepare the binder solution and transfer it into the high shear mixer.; Transfer Pump is required and shall be integrated with the system. Solution preparation tank should be mobile.. IBC Lifter; To lift dispensed material and charge into RMG; STS capacity. Rapid Mixer Granulator; To blend powders with a binder solution using high-speed mixing and shear forces to form granules; Supplier to provide the following bowls - 250L – 300L bowl - Provisional outlet to install additional wet milling in the future - Platform ladder should be detachable and movable. Wet Mill; The product to be milled comes from the discharge of the high shear mixer equipment and is fed to wet mill for milling process.; Two sets of screens shall be supplied by vendor.. Fluid Bed Processor; - To dry the granulated product transferred from high shear mixer. - For products made using top-spray granulation, the raw materials are transferred straight into the Fluid Bed Processor (FBP) using a Polybottle/IBC Bin. The FBP carries out both granulation and drying in one step; Hopper station will collect dried granules prior to milling. Dried granules are transferred from FBP to dry mill from the hopper. Supplier to provide the following - 250L – 300L bowl - 5 sets of antistatic finger bag. Hopper Station; Applies vacuum to transfer dried granules from the FBP to the next process stage.; Hopper size should be of sufficient size to collect all dried granules prior to dry milling. The hopper should have independent inlet/outlet valves so that the dried granules would not be going directly to the conical mill from the FBP. Antistatic finger bags shall be supplied in 5 sets. The hopper should be wall-mounted and height-adjustable.. Dry Mill; The Dry mill is used for milling the dry product before the dry product is sent to the IBC for collection.; The hopper station is connected to the dry mill. The screen size to be used varies depending on the product and should be supplied in five sets. The hopper should be wall-mounted and height-adjustable.. Loading of Materials through Split Butterfly Valve Assembly; Split butterfly valve with compensator on high shear mixer granulator and fluid bed processor inlets. Discharge of products from dry mill to IBC is through split valve assembly.; Closed transfer of powder material into the system from IBC to high shear mixer granulator, fluid bed processor and dry mill assembly. Integration of split butterfly valves shall be in wet granulation supplier’s scope.. Air Handling Unit, Dust Collector and Scrubber Unit; Inlet and exhaust air requirements shall be fulfilled by this unit and the dust could be collected in the bag-in and bag-out unit. The organic vapor traces are treated in wet scrubber.; N/A. CIP/WIP Unit; To wet, wash down and washing -in – place the high shear mixture, fluid bed processor and associated auxiliary equipment.; Equipment should be equipped with rotating actuator spray balls for cleaning. All ancillary hoses for cleaning shall be supplied. CIP for each system should be able to run parallelly/separately and independently of each other. Equipment should be equipped with WIP for auxiliary equipment such as deduster and supply and return ducts. Systems control should be provided on separate HMIs than the equipment HMIs.. IBC Bin; To blend powders and granules.; 5 sets of 500L IBC bins that can integrate with existing system (frame and height).. |

| 5.2 | PROCESS PARAMETERS AND ASPECTS |  |  |
| --- | --- | --- | --- |
| 5.2.1 | Critical Process Parameters, CPP’s |  |  |
|  | Rapid Mixer Granulator, RMG |  | - Impeller speed - Chopper speed - End point measurement (Amperage) - Cycle duration - Peristaltic Pump Speed and Binder Addition time |
|  | Wet Mill |  | - Speed - Screen size – 10mm |
|  | Fluid Bed Processor, FBP |  | - Inlet air temperature and humidity - Outlet air temperature - Product temperature - Differential pressure in FBP (3 sensors): - Location of sensors shall be specified by supplier - Inlet airflow - Spray rate of granulation liquid - Spray pressure - Cycle duration - Atomizing air pressure - Spray pattern (as option) |
|  | Dry Mill |  | - Speed - Screen size |
|  | General |  | - HMI / PLC should be designed with 21 CFR Part 11 compliance software. - The equipment should be designed in containment system up to OEB 5 level. - Loading and discharge stations should be suitable for attachment of buck valve (151mm). - Equipment should be provided with BIBO filter. - The equipment should be compatible with CIP cleaning. - Electrical, cables, pneumatic hydraulic and instrumentation equipment shall be identified and correspond with Electrical and Instrumentation drawings. |
| 5.2.2 | Critical Design Attributes, CDA’s |  |  |
|  | General | - The equipment and its parts must be easily cleanable. - Electrical, cables, pneumatic hydraulic and instrumentation equipment shall be identified and correspond with Electrical and Instrumentation drawings. |  |
| 5.2.3 | Critical material Attributes, CMA’s |  |  |
|  | General |  | - All outer surfaces of product contacting parts should be of pharmaceutical grade stainless steel AISI 316L with a suitable surface roughness (Ra <= 0.4 μm). - The inside surface of the equipment (product contacting) should be mirror polished with a lower surface roughness (Ra <= 0.4 μm) - Other product contacting surface should be made up of stainless steel AISI 316L, mirror polished and with a surface roughness of Ra <= 0.4 μm. - Non-contact part should be of pharmaceutical grade like non-rusting, non-shedding, non-corrosive, or SS 304. - Process materials should not react with equipment, product contact parts and MOC certificates required for all contact parts should be provided. |
| 5.2.4 | In-Process Controls |  |  |
|  | General |  | - The sampling port should be designed for in-process LOD testing. - The system should have batch data/report after batch completion for viewing, reviewing and printing. Reports should be in both a Human readable (PDF) format and un-readable format to back up and restore. |
| 5.3 | PROCESS FLOW |  |  |
|  | Wet Granulation in HSM and dry in FBP step. - Binder is prepared in the solution preparation tank - Powder is charged into the Rapi Mixer Granulator (RMG) for mixing. - Binder solution is added to form wet granules. - Wet granules are transferred to the Fluid Bed Processor (FBP) for drying. - Sampling ports are available on the FBP for sampling of dried material. - Dried granules are discharged from the FBP to the hopper - Dried granules charged through dry mill and collected in an Intermediate Bulk Container (IBC). - A new batch can be charged into the RMG while the FBP is drying, enabling a continuous process without waiting for FBP completion. - After completion, Washing-in-place module can be executed at each station parallelly and independent of the other stations. - Below the processing with RMG & FBP for granulation and drying steps. |  |  |
|  | Top spray granulation and drying steps. - Binder is prepared in the solution preparation tank - Powder is charged into the FBD. - A spray lance with an atomizing nozzle is fitted in the processing chamber and used for top spray granulation and drying. - Sampling ports are available on the FBP for and dried material. - Dried granules are discharged from the FBP to the hopper - Dried granules charged through dry mill and collected in an Intermediate Bulk Container (IBC). - Below is the processing with Top Spray granulation and drying.. |

| 6.0 | CLEANING REQUIREMENTS |
| --- | --- |
|  | Cleaning-In-Place. Req No; Description; Equipment operating range/value. 6.1; CIP unit shall follow the 4 cleaning phases steps as follows - Wetting with hot potable water - Washing with SLS - Removal of SLS with RT potable water - Final rinsing with purified water; The cleaning recipe with automated sequence as mentioned is included.. 6.2; A typical CIP cycle with peak loads shall be suggested by supplier for this process module.; N/A. 6.3; Product filters on the FBP shall be wet prior lifting down and manually removed before the full CIP cycle.; N/A. 6.4; Wash down of the systems both high shear mixer and fluid bed processor shall be through washing-in-place unit with option for one detergent dosing (Dodecyl sulfate sodium salt). shall be planned.; STS. 6.5; The CIP time shall be registered. The cleaning process must be documentable.; Total cleaning time should not exceed 3 hours.. 6.6; Prior to launch of CIP Cycle, some utility and drain connections could be done manually by the operator.; N/A. 6.7; The CIP unit shall provide connections and spray nozzles in locations as needed to effectively wet, wash down and clean the different equipment in the suite.; N/A. 6.8; Wetting efficiency shall be verified in FAT or SAT using Riboflavin solution test.; STS. 6.9; CIP media shall be supplied with controlled and registered temperature.; N/A. 6.10; After the last rinse, the equipment will be completely dried using compressed air for RMG, FBP, Dry Mill, and all ducts.; N/A. 6.11; After the CIP process is ended, the equipment must be clean and dry and shall be controlled visually.; N/A. 6.12; The full CIP system shall be designed in a sanitary way (easy to clean, no dead-legs ); STS including of rotating actuator spray ball. 6.13; The CIP system and the HMI should be integrated with the existing CIP system already in place at Novugen (as applicable); N/A. 6.14; CIP shall be carried out for the IGL systems as follows: - Equipment CIP: RMG, Wet Mill, FBP, Hopper, and Dry Milll) - Ducts: Supply duct and exhaust duct (full duct cleaning) - Deduster: full deduster cleaning; N/A. |

| 7.0 | GENERAL REQUIREMENT FUNCTIONS |
| --- | --- |
| 7.1 | Functions |
|  | - Data Security And Other Controls - Operator – Provides operator access to allow routine operation of equipment features. - Supervisor – Provides supervisor access to allow routine operation of equipment features and create recipes. - Maintenance – Provides maintenance access to allow routine operation of equipment features. Should not be able to access or modify any recipes. - Administrator – Provides access to the Operator, Supervisor and Maintenance level features in addition to system security parameters and create new users. Able to delete recipes. - System software shall provide security to prevent unauthorized parameter changes. - Recipes shall only be archived by the system administrator. - The system shall provide the ability to utilize built-in lock-out policies to automatically log -off the end user (e.g., Inactivity time). Hard copy / electronic data collection requirements. - The system should use complex and encrypted passwords for users. - The audit trail must include the changed action, user ID, date, time stamp, old value, and new value for each activity and shall not be limited to batch manufacturing only. - The system must provide functionality to allow all auditable records, including the audit trail log itself, to be accurately and completely copied electronically or printed. - The audit trail must be protected from modification and the audit trails report must be possible to print/export to PDF, in human readable format. - All previous versions of auditable records must be retained. The system must generate an electronic audit trail that is always active for all user activities that create, modify, or delete records. - The system must have backup and restoration capabilities. - The system must allow the configuration of multi-level security accesses to menus and data according to the needs of the site, to specific individuals following their job responsibilities scope. - The system must only be accessible by authorized users. - Exporting data (PDF) through an Ethernet connection is possible on an ad hoc basis. - The system must provide means to protect data from hard drive failure. - The system must follow the data security guidelines of Novugen Oncology that shall be provided. - The system shall operate and be accessible exclusively on the production local network by authorized personnel only. - The system should require secure internet access, and it should be controlled via allowing data traffic access to specific IP and ports, and with authenticated and secure connection. - The users shall not be able to change the system's date and time. - The system must have the ability to grant access to a new user, changing privileges for an existing user and deactivating user accounts with proper access level. - The system must be able to prevent unauthorized user access to the system and record all events in the audit trails. - The system must prevent any user from manually updating the data within the system (i.e., batch data, recipe data, audit trails). - The system must perform user authentication that allows only authorized and active individuals to access the system by using a unique user ID and password. - The system must support password maintenance as per Novugen Oncology security standards and group policies as prescribed in the SOP for Control of IT Systems and Password Management. - The system must lock users out after a predetermined number of unsuccessful attempts. - The users are not allowed to use the last 5 recent passwords. - The system must automatically log out users after a pre-determined period of inactivity. - The system must continue to operate as an equipment non-human operator if user time out occurs. - The system must only allow the unlocking of an account by system administrators. - The unlocking of accounts must be able to be performed remotely. - The system must only allow administrators to deactivate users in the system. - The system must display login errors when user identity or password cannot be authenticated. - The system must provide remote access to the system for OEM to support via remote access software. - The system must capture in the audit trail when the security level of a user account is modified. - Only the Administration must have access to the operating system. - After power-up, the system shall open the application directly without going to operate the system. - The system should employ secure, computer-generated date and time stamped audit trails to independently record operator entries and actions that create, modify, or archive electronic records, without obscuring previously recorded information. These include, username, date and time stamp, old value and new value, indication of record or file creation, modification or archival, and capture batch comments. - The system must be able to prevent the audit trail from being modified or deleted. - The system shall restrict any user to disable the audit trail function. - The system must be able to print an audit trail report in human readable non-modifiable format on demand. - Storage of records shall remain in the systems until archived. - Supplier shall ensure that systems and data backups are continuously maintained during the system validation, for effective restoration if required. - The vendors shall provide step by step detailed documentation on systems and data backup procedures, frequency of backups, source paths of backups, systems, and data backup restorations procedures. - Upon restoration, the system shall ensure the data are accessible and shall be executed similar to its previous history. - The system must have access privileges. Vendor shall provide the machine roles for every access level. - System must have provision to create / modify the recipe up to supervisor level. - The deletion/deactivation of recipe shall be applicable to Administration only. - The system user operations manual and administration manual shall be provided by the vendors. - All moving parts shall be guarded and secured with an interlocking system. - The system will have an E-stop mechanism designed to stop all physical movement of the equipment immediately. The E-stop mechanism(s) must be in easily accessible areas around the equipment as required by national and local safety standards. - The installation and operation of the equipment shall ensure the ergonomic aspect of staff to be handled. - The manufacturer shall advise Novugen Oncology staff if there is any other safety system that may be implemented and if there is any PPE required during handling the machine. - The system shall be capable of continuous operation without requiring major maintenance and during maintenance windows as well. - The resiliency shall be designed in the system infrastructure to provide high availability and ensuring zero down time. - Accessibility for easy maintenance, for maintenance activity, required spares, etc. - The vendors shall provide a technical manual that describes the procedure in detail for troubleshooting. - The service technician should be fully conversant with the system and support in case of any problem. - The lifetime of software must be stated. - The vendor shall provide proven and tested software versions. The software QA reports may be requested, when required. - The systems, including hardware and software, shall be maintained by the supplier by periodic maintenance and software upgrades and patching. - There must not be any expiry date for the hardware and software components provided. - The Vendor should provide software media on USB or CD-ROM, as appropriate, and ready to install if required. - The vendor engineers shall be available on site and engage during PQ to provide seamless support. 21 CFR Part 11 Requirements. - The software supplied for the system shall comply with the PIC/S Guideline and US FDA CFR21 Part 11. - The system shall generate the daily full backup and incremental backup automatically. - The live data storage and backup location shall be different. - The system shall be able to transfer backup data on defined network location. - The system shall prompt to archive data on specified capacity. - The system shall restore the backup taken. - The system shall allow us to generate accurate and complete copies of data in both human readable and electronic form. - The system shall restrict unauthorized users from accessing the system. - The system should have at least 4 user groups i.e., Operator, Supervisor, Maintenance, and Administrator with different rights to each role (maximum rights to administrator). - The system shall have unique username functionality. - The system shall have password complexity for e.g. password length 8-character, password expiry 90 days, password masking, prompt to change the password at first time login. - The system shall restrict the use of the function according to pre-configured user profiles that are maintained i.e Authority check shall follow the sequence. - The system shall log off automatically in ideal condition or 15 minutes of system inactivity. - Only administrator shall have the rights to go to operating system of the machine. - Only administrator shall have the rights for user management and access any drive-in system. - System shall generate the reports in non-editable format. - The system shall synchronize the time from operating system locally or network (if any). The system shall use device checks to determine the validity of source data from other devices i.e. device checks. - The system shall record the alarms, events, user login /logoff and other information in audit trail with time/date stamp and old/new value with reason of change. - The system shall maintain the Help menu/ application information with current version. - The system shall prompt the power failure message in case of power failure to shut down the application in efficiently manner to avoid the data loss. - The system shall save the data (buffering) in case of network and device communication.. |
| 7.2 | Control functions for operational parameters |
|  | 7.2.1 Control Functions for Operational Parameters. Sr. No.; Description; Equipment operating range/values/units. Wet Granulation Suite: The below stated functions and the minimum, but not limited to, required functions to be controlled via the HMI and controls.. Rapid Mixer Granulator and Wet Mill. 7.2.1.1; Spray Rate; g/min. 7.2.1.2; Spray Pressure; bar. 7.2.1.3; Quantity granulation liquid; Liter or gram.. 7.2.1.4; Solution Pump Speed, if peristaltic pump:; 0-100%. 7.2.1.5; Low level alarm; N/A. 7.2.1.6; Spray pattern; bar. 7.2.1.7; Impeller speed; r/min. 7.2.1.8; Impeller tip speed; m/s. 7.2.1.9; Impeller power consumption; amperage. 7.2.1.10; Chopper speed; RPM. 7.2.1.11; Chopper power consumption; amperage. 7.2.1.12; Process Time Trip; min. 7.2.1.13; Product Temperature Trip; °C. 7.2.1.14; Impeller Load Trip Level; kW/aperage. 7.2.1.15; Speed of wet mill impeller; RPM. Fluid Bed Processor and Dry Mill. 7.2.1.14; Fluidising air flow; m³/h. 7.2.1.15; Dehumidifier Temperature; °C. 7.2.1.16; Inlet air temperature; Temperature: Ambient to 80°C. 7.2.1.17; Inlet air humidity; <10g of water/ kg of air. 7.2.1.18; Outlet air temperature; Temperature: 30°C to 80°C. 7.2.1.19; Outlet air humidity; Optional. 7.2.1.20; Product temperature in fluid bed; Temperature: 30°C to 80°C. 7.2.1.21; Air flow (transfer air flow – HSM – wet mill to FBP); STS. 7.2.1.22; Temperature (transfer air flow – HSM- wet mill to FBP); STS. 7.2.1.23; Delta P product bowl; Pa / m bar. 7.2.1.24; Delta P FBP product filters; Pa / m bar. 7.2.1.25; Time duration for each process in general for HSM & FBP process in series of FBP only - Loading - Granulation - Drying - Discharge; Batch running time will be displayed on HMI. 7.2.1.26; Spray rate (during granulation process); Approximately 1000 in g/min. 7.2.1.27; Spray pressure (during the granulation process); Bar (g). 7.2.1.28; Spraying flow control/shut off; Blow back for filter shall be considered. 7.2.1.29; Speed of dry mill impeller; RPM. 7.2.1.30; Process Time Trip; Min/sec. 7.2.1.31; Endpoint Operation; Time/Heating. 7.2.1.32; Exhaust Temperature Trip; °C. 7.2.1.33; Start/stop; N/A. 7.2.1.34; Emergency stop; N/A. 7.2.1.35; Reset; N/A. Cleaning in place (CIP):. The below stated functions are the minimum, but not limited to, required functions to be controlled via the HMI.. 7.2.1.32; CIP media pressure wash fluid supply; 2-6 bar. 7.2.1.33; CIP media flow nominal flow; m³/h. 7.2.1.34; CIP media temperature(inlet); 20-80 °C. 7.2.1.35; CIP media concentration; 0-10%. 7.2.1.36; Drying air (inlet air); 30-80 °C. 7.2.1.37; Detergent dosing pump; STS. 7.2.1.38; Low level alarm for detergent tank; STS. 7.2.1.39; CIP pump flow; N/A. 7.2.1.40; Conductivity – final rinse(optional); Included in wash skid. 7.2.1.41; Start/Stop; N/A. 7.2.1.42; Emergency stop; N/A. 7.2.1.43; Reset; N/A. |

| 7.3 | Control System |
| --- | --- |
|  | 7.3.1 General Control System Functions. Req No; Requirement; Comments. 7.3.1.1; Control system; PLC based. Siemens preferred by customer supplier to advise on the makes.. 7.3.1.2; Human Machine Interface - HMI; Touch panel (Colour, min 10 inch) IP65.. 7.3.1.3; Control Functional Specification (FS); To be shared by supplier. 7.3.1.4; The system shall have a password protected screensaver or similar safeguards after a specified period of inactivity. HMI shall be programmed for auto logout.; N/A. 7.3.1.5; A default “User” (not included in a user group) shall be created. This “User” is logged in at start up and must only have visualization rights to all pictures.; N/A. 7.3.1.6; The system shall contain a recipe handling tool, change in recipe must be logged by user with username and password.; STS. 7.3.1.7; The system shall comply with GAMP 5.0 methodology at a minimum, validated that supplier provides this customer with the functionality, process and technical controls its needs to help comply with regulatory standards including US FDA Title 21 CFR Part 11; N/A. 7.3.1.8; The system shall ensure that only authorized individuals can: - - User the system - Access the operation or computer system input or output device Perform the operation at hand; N/A. 7.3.1.9; IP Address configuration and setting of Baud Rate; N/A. |
|  | 7.3.2 Batch Report. Req No; Description. The system shall be able to create a batch report saved locally in a memory card; after each cycle to document the executed process. The batch processing report shall be created with the following minimum requirements but could be elaborated by the supplier.. 7.3.2.1; Date. 7.3.2.2; Start and stop time for the recipe. 7.3.2.3; Product Name. 7.3.2.4; Batch Number. 7.3.2.5; All recipe values. 7.3.2.6; All executed values (Process data – table with actual set point for all minimum and maximum values). 7.3.2.7; All login interactions/interventions. 7.3.2.8; All events. 7.3.2.9; All messages. 7.3.2.10; All measurements & graph (the set point of measured parameter needs to be displayed on the graph). 7.3.2.11; All alarms. 7.3.2.12; Operator ID. 7.3.2.13; PDF batch records shall be generated at batch end and shall not be modifiable.. 7.3.2.14; Excel batch records so the user can extract the data compilation.. |
|  | 7.3.3 Back Up and Data Management. 7.3.3.1; The historical data of the system shall be stored in dedicated server /suppliers control system. Historical data should be accessible and available for trending analyzing.. 7.3.3.2; Systems shall be able to make data backup automatically or manually. The data backup includes all raw data, configurations and audit trails records.. 7.3.3.3; System shall be able to support to archive backup database to external storage.. 7.3.3.4; The system shall be able to retrieve and restore data from the backup database.. 7.3.3.5; The supplier shall design the system to include a “safe state” in which the likelihood injury to personnel and damage to the equipment is minimized.. 7.3.3.6; In case of a power failure, the system shall be able to retain the data in a safe state. Upon power restoration, the system shall be able to start by itself and recall all controlling parameters. A “Safe state” shall be defined as : All motion stopped on the machine valves and actuating devices go to safe state and to keep containment intact (TBH by supplier).. 7.3.3.7; Vendor shall supply UPS with the system for the purpose of power back up.. |

| 8.0 | INSTALLATION |
| --- | --- |
| 8.1 | Positioning and dimensions |
|  | Sr. No.; Requirement. Approved Equipment Layouts shall be considered for actual positioning of the process modules. Space for utility connections and maintenance shall also be taken into account in the drawings.. 8.1.1; Positioning area (area under scope of Novugen Oncology). 8.1.2; The supplier shall provide with the quotation general arrangement layout of the complete in granulation suite including technical equipment and critical dimensions for equipment entrance and maintenance. |
| 8.2 | Installation Requirements |
|  | Sr. No.; Requirement. Below the requirements are separated into general requirements and requirements for each process module or unit.. 8.2.1; Framework and carriages shall be made of stainless steel SS304L/SSS304 or better.. 8.2.2; In general, the equipment has to be designed in a way and quick access to all necessary maintenance points e.g. pumps, motors, filters, valves, instruments etc.. 8.2.3; ATEX classification. The supplier shall provide the ATEX classification of its equipment.. 8.2.4; All product contact parts must be made of stainless steel, AISI SS316L with surface grade Ra ≤ 0.4 mm. Materials certificates will be supplied for all parts that are exposed to the product EN 10204-3.1.1 The vendor will deliver certificates of roughness for all product contact parts. Supplier to provide all manual conformity certificates.. 8.2.5; All non-product contacts parts must be made of stainless steel, AISI SS304L/SS304.. 8.2.6; All gaskets to be food grade such as silicon, PTFE or EPDM rubber free of asbestos and need a FDA approval/TSE/BSE Certificates for compliance.. 8.2.7; All material must be resistant to be cleaning agent and disinfected.. 8.2.8; All lubricants of the equipment have to correspond with EC and FDA requirements for lubricants used for parts with food contacts.. 8.2.9; All welds to be crack and crevice free to provide a smooth internal surface.. 8.2.10; Rounded angles for easy washing / drying (all welds to be crack and crevice free to provide a smooth internal surface.). 8.2.11; The slope for piping and horizontal surface has to be suitable flow (all welds to be crack and crevice free to provide a smooth internal surface.). 8.2.12; RMG bowl to have 0.22 µm hydrophobic vent filter/equivalent.. 8.2.13; Filter step I (fine-dust filter) F9 required in exhaust.. 8.2.14; Filter step II(HEPA) EU-14/H13 required in exhaust.. 8.2.15; Differential pressure sensor with switch/transmitter for the inlet air pre-filters, current values shall be indicated (local pressure measurement). 8.2.16; Exhaust / outlet air handler.. 8.2.17; Differential pressure sensor with switch/transmitter for the inlet air pre-filters, current values shall be indicated (local pressure measurement). |
| 8.4 | Requirements for buildings |
|  | Sr. No.; Requirement. Process module comprises of HSM, FBP with air handling unit, dust collector and CIP unit to be installed at ground floor processing area in this manufacturing building.. 8.3.1; In granulation line and dust collector are to be installed in clean room (Grade D area), through the wall design.. 8.3.2; Air handling unit and CIP unit to be installed in the technical area.. 8.3.3; Free fresh air supply is considered in the technical area where the air handling unit is located (Supplier shall specify the quantity of air to be supplied in m³/h). 8.3.4; Supplier shall specify the dimension of the exhaust air of the following: Dust extractor, air handling unit.. 8.3.5; Wall portioning shall be installed after the installation of the equipment (especially with installation through the wall). Finishing (silicone, airtightness) shall be ensured by Novugen Oncology.. |
| 8.5 | Utility Connections |
|  | Connection of utilities from point of use to the process equipment and auxiliary equipment shall be in Novugen Oncology scope of work. Supplier to provide suitable end connections to connect utilities. Electric cable delivery nearby the electrical cabinet done by Novugen Oncology. Connection of the cable from the cabinet to the equipment shall be in the scope of supplier. The layout/ design shall be provided by supplier after discussing with Novugen Oncology engineering and production teams, ensuring all equipment physical integrations requirements are planned beforehand and delivered before the equipment delivery at Novugen site. Supplier to consider the exhaust air piping, electrical wiring, and piping length in actuals during the installation from main to the auxiliary equipment. The scope of supply to be clearly mentioned in the P&ID and all relevant drawing submitted for approvals. Service; Supply Conditions; Average Operating Load; Peak Load. Electricity; 3 Phase; 415 Volts ± 10%, / 230 V Frequency: 50Hz; To be advised. Compressed Air; Process air Pressure: 8 bar Instrumentation air: 8 bar; Operating pressure required: STS Operating temperature required: STS; STS. Chilled water for air handling unit; Supply Pressure: 2 - 4 bar Supply temperature: 7°C to 9°C; Operating pressure required: STS Operating temperature required: STS; STS. Steam; Supply Pressure: ≤ 3.5 bar Supply temperature: approximately 140°C; Operating pressure required: STS Operating temperature required: STS; STS. Potable water ambient; Pressure: 1 - 3 bar Temperature: Ambient; Operating pressure required: STS Operating temperature required: STS; STS. Potable hot water; Pressure: 1 - 3 bar Temperature: 50°C -70°C; Operating pressure required: STS Operating temperature required: STS; STS. Purified water; Pressure: 1 - 3 bar Temperature: Ambient; Operating pressure required: STS Operating temperature required: STS; STS. |

| 9.0 | HEALTH, SAFETY AND ENVIRONMENT |
| --- | --- |
|  | 9.1 Health, Safety and Environment. Sr. No.; Item; Requirement. 9.1.1; All electrical wiring shall be concealed and with proper earthing arrangement; All these must be provided by Vendor during equipment design. 9.1.2; The noise level shall meet the local standard; <85dB. 9.1.3; Moving parts shall be guarded and secured with interlocking system; All these must be provided by Vendor during equipment design. 9.1.4; Have an E-stop mechanism designed to stop all physical movement of the equipment; All these must be provided by Vendor during equipment design. 9.1.5; Provide personnel training of operator and maintenance Engineer, validation execution, fast maintenance and guarantee against damage; All these must be provided by Vendor during equipment design. 9.1.6; The ergonomic aspect of staff to handle; All these must be provided by Vendor during equipment design. 9.1.7; Safety signages, Interlocks, flame proof electrical fittings and earth grounding shall be ensured.; All these must be provided by Vendor during equipment design. 9.1.8; All the installation must be in accordance with cGMP compliance; All these must be provided by Vendor during equipment design. |

| 10.0 | DOCUMENTATION / TRAINING |
| --- | --- |
| 10.1 | Required Documents: |
|  | Documents should provide by the Suppliers:. Sr. No.; Component/Features; Description. 10.1.1; P&ID / GA drawing; Required. 10.1.2; I/O details; Required. 10.1.3; Control system/Electrical drawing; Required.. 10.1.4; Power supply configuration arrangement drawings; Required.. 10.1.5; MOC certificates; Required.. 10.1.6; Interconnection diagrams; Required.. 10.1.7; System architecture; Required.. 10.1.8; Bill of materials; Required.. 10.1.9; Component list; Required.. 10.1.10; Component datasheet; Required.. 10.1.11; Instrument list; Required.. Commissioning documentation. 10.1.13; FAT documentation; Required. 10.1.14; SAT documentation; Required. 10.1.15; DQ, IQ and OQ protocol; Required. 10.1.16; PQ support; Required. 10.1.17; Calibration certificates; Required. 10.1.18; Test certificates; Required. 10.1.19; OEB5 Containment Declaration Letter; Required. - The DQ, IQ, and OQ shall be completed by the vendor along with Novugen Oncology representative. The supplier shall demonstrate the performance of the system at the user site as per agreed terms. - PQ will be performed and completed by Novugen Oncology. The vendors shall provide on-site technical assistance as required during the PQ. - The vendors shall specify the date of delivery from the time the order is placed. - The vendors shall specify the date of onsite installation and commission from the date of delivery equipment, as part of the project management schedule that shall be kept updated during the project lifecycle. - The vendor is responsible for commissioning equipment once installation is complete; and shall remain available on site during and till completion of first commercial batch, ensuring smooth operation and handover of the system. - The design and construction of the equipment must be such that sharp angles and corners and other parts, not easily accessible for cleaning, are avoided and free from any ink or oil leakage. - The equipment changeover should be clearly stated in the user manual. - Any special tool for cleaning processes shall be provided by the manufacturer. |
| 10.2 | Training |
|  | The Supplier shall provide a detailed and training for Operation and maintenance procedure. All equipment shall allow for a tool-less conversion wherever possible. If any equipment requires special (un-common hand-tools) tools for changeover or common maintenance, these tools (two sets) shall be supplied with the equipment and cost shall be contained in the base price. Sr. No; Requirements. 10.2.1; The machine shall be easily accessible to perform maintenance and service. 10.2.2; All possible adjustments on mechanics and instrumentations shall be defined and described in the documentation. 10.2.3; Proposal for service contract for periodic maintenance service shall be included as an option in the quotation. 10.2.4; The maintenance personnel shall have the necessary training in operation and maintenance. A training program shall be included as an option in the quotation. 10.2.5; A list of excepted service life and recommended service intervals of critical components shall be supplied. 10.2.6; All training and documentation regarding training shall be done in the English language. 10.2.7; Provide personnel training of operator and maintenance Engineer, validation execution, fast maintenance and guarantee against damage. 10.2.8; Supplier to provide warranty from the date of installation as agreed with customer during order finalization. 10.2.9; Accept on-site audit by Client. |

| 11.0 | REFERENCES AND DEFINITIONS |
| --- | --- |
| 11.1 | Reference Documents |
|  | 21 CFR Part 11; Code Of Federal Regulations 21 Part 11 Final Rule: Electronic Records; Electronic Signatures. QA Reference; Volume 4 of “The rules governing medicinal products in the European Union” Annex 11 Computerized Systems. EHS Reference; Compliance with the Safety, Health, and Welfare at Work Act. EHS Reference; EC Declaration of Conformity for equipment and machinery in accordance with EC Council Directive 98/37/EC. EHS Reference; Compliance with the Low Voltage Directive 72/23/EEC. |

| 12.0 | ABBREVIATIONS: |
| --- | --- |
|  | Abbreviation; Full Form. IBC; Intermediate Bulk Container. FBP; Fluid Bed Processor. GA; General Arrangement. 1D/2D; One Dimensional/ 2-Dimensional Data Matrix Bar Code. GMP; Good Manufacturing Practice. Max.; Maximum. min.; Minute. mm; Millimeter. NA; Not Applicable. No.; Number. MOC; Material of Construction. FAT; Factory Acceptance Test. SAT; Site Acceptance Test. HMI; Human Machine Interface. GTIN; Global Trade Item Number. GS1; Global Standard One (Normalization Organization). IQ; Installation Qualification. OQ; Operational Qualification. Sr No.; Serial Number. EPCIS; Electronic Product Code Information Service. URS; User Requirements Specification. CFR; Code of Federal Regulations. HMI; Humans and machines to interact. FS; Functional Specification. DS; Design Specification. OEM; Original equipment manufacturer. QA; Quality Assurance. |

| 13.0 | REVISION HISTORY: |  |  |
| --- | --- | --- | --- |
|  | Rev. No: | Effective date: | Reason for revision |
|  | 00 |  | New URS |

| 14.0 | VENDOR ACCEPTANCE The User Requirement Specification has been discussed and agreed upon. We hereby declare that we will supply the System/Equipment /Instrument/Software as per above laid down specification. |  |
| --- | --- | --- |
|  | Name of the Vendor | Signature / Date |